Guide to common errors encountered with the AMT7450 Datasouth printer:

Head Protection:

- Description: Print head becomes too hot during high-density printing.

- Resolution: Reduce printing speed or wait for print head temperature to drop.

Park Error:

- Description: Fanfold paper not fed back to the parking position properly.

- Resolution: Remove and reload paper.

Paper Out:

- Description: Fanfold paper not loaded in rear tractor, or cut sheet not loaded into the printer.

- Resolution: Install fanfold paper in rear tractor or load cut sheet into the printer.

Paper Jam:

- Description: Paper jammed at tractor or false paper jam message.

- Resolution: Remove jammed paper or perform tractor Initialization 2. Set menu 26 PAPER JAM DETECT to NO if necessary.

Remove Paper:

- Description: Cut sheet did not eject properly.

- Resolution: Remove paper manually or wait until the end of print job for automatic ejection.

Skew Error:

- Description: Cut sheet loaded at an angle beyond printer's ability to square it.

- Resolution: Reload paper straight. Disable menu item 25 PAPER SKEW DETECT if necessary.

Cover Open:

- Description: Front cover opened during printing.

- Resolution: Close front cover to clear error. Press ONLINE key to put printer back online.

Jam Sensor:

- Description: Jam sensor detects paper jam.

- Resolution: Press ONLINE key to clear error. Ensure jam sensor on tractor is clear and clean.

This guide provides brief descriptions of common errors and their resolutions for the AMT7450 Datasouth printer. If issues persist, refer to the manufacturer's documentation or contact AMT7450 support for assistance.

https://www.amtdatasouth.com/support